Zachary Gold

NSF GRIP Intern

Division: FRD

Supervisor: Andrew Thompson

06/20/2019

Ichthyoplankton Ethanol Filtration Protocol

This was the protocol used for https://onlinelibrary.wiley.com/doi/pdf/10.1002/edn3.400

Preparation

- Set Up Filtration Manifold

  - Equipment:

    - Manifold

    - 6 stoppers with attached plastic connector funnels

    - Large 4L glass Erlenmeyer flask for ethanol collection (labeled 2)

    - Plastic Erlenmeyer flask as air trap (labeled 1)

    - Two Stoppers for Erlenmeyer flasks

    - Tubing

    - Adapters/connectors

    - GAST Vacuum Pump

  - Connecting Manifold

    - Attach air trap directly to pump

    - Attach ethanol collection to air trap

    - Attach ethanol collection to manifold

    - Test pump is working effectively and reaching at least -20 PSI

- Sterilize Work Station

  - RNAse Away + ethanol all surfaces

  - RNAse Away + ethanol items to be handled when filtering

    - I.e. pens, racks

- Lab Equipment Needed

  - Fresh 50mL Tube with 100% bleach (change each 6 samples)

  - Fresh 50mL Tube with 100% ETOH (change each 6 samples)

  - Forceps (at least 2)

  - Lighter (NOTE: FLAME FORCEPS OVER SINK)

  - Pre-label 2mL screw cap tube for each sample

  - Pre-label manifold with lab tape for each sample

  - Filter Funnel for each sample

  - Durapore PVDF Filters

  - Freezer Box for sample storage

  - Ethanol waste bin

  - Tris buffered ethanol for replacement

- Ichthyoplankton samples

  - Invert Ichthyoplankton samples 3 times at least 30 minutes before sample use

  - Allow time for ichthyoplankton to re settle

  - Invert next round of 6 samples ahead of time

Filtration

- Attach 6 single use filter funnels to manifold

- Sterilize forceps: bleach, ethanol rinse, flame over sink

- Remove plastic cap of filter funnel, lay upside down on plastic lid.

- Dispose of top CN filter using sterile forceps, DO NOT remove the fuzzy lower filter

- Using sterile forceps, add new PVDF filter

- Replace plastic lid, making sure filter is centered on the plastic lid and sealed tightly

- Mark filter funnel with FR (filter replaced)

- Repeat Steps 2 through 7 sterilizing forceps each time until all filters are replaced

- Bring 6 samples from specimen room to eDNA room

- Pipette top 200mL of ethanol from each sample into each respective filter funnel.

  - DO NOT CAPTURE ICTHYOPLANKTON

  - If sample has multiple jars and >200mL ethanol, add just the first 200mL

- Turn on pump to begin filtration

  - Operation of Pump

    - NEVER OPERATE PUMP WITHOUT AIRFLOW

      - Double check that AT LEAST ONE valve is open when pumping

    - NEVER ALLOW LIQUID ON PUMP

    - NEVER ALLOW LIQUID TO FLOW INTO PUMP

      - Double check volume of waste ethanol within collection flask

      - Ensure tubing into pump is free of moisture and liquid

- Once all ethanol is transferred through the filter, turn off valves, then turn off pump

  - MAKE SURE AT LEAST ONE VALVE IS OPEN when shutting off the pump

  - DO NOT RUN PUMP ON DRY FILTER FOR TOO LONG

- If sample has >200mL, turn off valves for all other samples, turn off pump

  - Add remaining ethanol from samples to the filter funnel

  - Turn on pump to finish filtering those samples

- Next sterilize forceps: bleach, ethanol rinse, flame over sink

- Gently remove and dispose of plastic filter cup, DO NOT SCRAPE THE FILTER WITH THE CUP

- Open screw cap filter, leave cap upside down on sterile surface

- Use sterile forceps to tightly roll PVDF filter

- Transfer filter to 2mL screw cap, and seal

- Repeat steps 14-18 for all 6 samples

- Transfer 2mL tubes to freezer box

- Store at -20˚C until DNA extraction

  - Extractions should be completed within 1 week of filtration.

- Return samples to specimen room

- Refill samples with Tris-buffered ethanol

Preparation for next Filtration

- Discard and replace bleach and ethanol 50mL tubes

- Check Erlenmeyer flask for volume of ethanol waste.

  - If over 2L transfer to waste ethanol container

- Sterilize surfaces

- Re-label manifold for new samples

- Invert next round of 6 samples if necessary

- Take out trash if necessary

- Run vacuum open at the end of the day for maintenance